Faculty of Computer Technology and Cybersecurity

Department of Computer Engineering

ARCHITECTURE AND ORGANIZATION OF COMPUTER SYSTEMS

LABORATORY WORK - 3

Date: 26.09.2026

Full Name: Daniil Kalts

Group: IT2-2404SE

PROGRESS OF WORK

1) Start the Microchip Studio.

2) Create a new GCC C Executable project.

3) Select the device (in this work — ATmega2561.

4) Next, program the microcontroller.

#define F_CPU 8000000UL #include <avr/io.h> #include <util/delay.h> void pwm_init(void) { DDRB ¦= (1 << PB7); TCCR0A = (1 << WGM00) ¦ (1 << WGM01) ¦ (1 << COM0A1); TCCR0B = (1 << CS00); OCR0A = 0; } int main(void) { pwm_init(); DDRA &= ~(1 << PA0); PORTA ¦= (1 << PA0); int brightness = 0; int direction = 1; while (1) { if (!(PINA & (1 << PA0))) { brightness += direction; if (brightness >= 255) { brightness = 255; direction = -1; } else if (brightness <= 0) { brightness = 0; direction = 1; } OCR0A = brightness; } _delay_ms(10); } }

5. Compile the project and obtain the .hex file.

6. Open Proteus (ISIS Professional) and pick the ATmega2561, Button, LED, Resistors, Power and Ground.

7. Load the compiled .hex file into the microcontroller's properties (Program File) and

configure the component

8. Run the simulation and verify that the LED smoothly brightens and dims while the button is held, then stays at its current brightness when the button is released.

Conclusion:
In this laboratory work, I programmed the ATmega2561 microcontroller in C and tested the circuit in Proteus. Timer0 generated a PWM signal on PB7 to control the LED brightness, while a button on PA0 controlled the fading. When I held the button, the LED smoothly brightened and dimmed; when I released it, the brightness stayed at its current level.